Less.3 ARP 3/10/2025

The robot must have components that take information from different sensors (such as lasers) to estimate position ecc…

All the processes in an architecture are concurrent (independent) and every process can access only a part of the memory (computational space).

About the homework_architecture:

*The MAP is a shared memory (blackboard) (accessible for all components) with obstacles, streets but also sensors, localization ecc..

*The connection between components and shared memory are in general bidirectional.

The publisher has many subscribers and is one direction communication: the subscribers only receive the information. (Serial data, for POSIX: serial binds)

We can connect to the POSIX model with FIFOs to push the messages.

Fifo

Publisher

If you take a snapshot you can’t know which information arrived at the subscriber because it can be slow and some data have not arrived yet.

The information that the publisher send are duplicates messages for each subscriber and are sent at the same time.

About the trajectory:

The planner decides the route in reference to the minimum VIA POINTS(points you must go to), it generates a graph. It also takes in account the speed the robot must have for turning, to top, to start again ecc..

The algorithm generates the full solution but during the execution it can be stopped at any time.

The trajectory real-time, that is connected to the wheels, controls them motion.

Every time the process is stopped, the planner backtracks and recalculates all the trajectory (takes time). For small obstacles it can just adapt the trajectory without backtracking. (continuous routine)

To summarize:

*planning (generate graphs of via points)

*trajectory modeller and trajectory adaptation (continuous laws of motors)

*trajectory real time (generates the wheels rotation)